| Style No. PBLT03 Phoebe Small Ceiling Mounted LED Flush Mount with Glass Diffuser | Style No. PBLT04 Phoebe Large Ceiling Mounted LED Flush Mount with Glass Diffuser |
| --- | --- |

SPECIFICATIONS:

Bulb Base Size: LED

Bulb Type: LED BOARD

Junction Box Orientation: Any

Junction Box Size: Standard 4: Octagon

Max Wattage: 12 Watt

Orientation: Ceiling, or Wall

Diffuser Material: Glass

Voltage: 120

Required Electrical Feeds:

US: Black “Hot” wire; White “Neutral” wire; Green or Bare

“Ground” wire

IMPORTANT:

- WARNING: The power supply must be turned off prior to installation. Turn off the main electrical supply to the junction box from the fuse box/circuit breaker. Turning power off at the switch is not sufficient.

- To ensure this product is installed properly, you must read and follow these guidelines.

- The owner/user of this product must keep this information for future reference.

- PBLT03/PBLT04 are U.L. certified and are suitable for damp or dry locations. The U.L. listing mark can be found on the inside of the cover plate.

- Be sure your installation conforms to all federal, state, and local codes.

- This product must be installed by a professional licensed electrical contractor.

- Refer to the specification and assembly drawings attached. Product is sold partially assembled but shown fully disassembled for illustrative and service purpose only.

PBLT03 / PBLT04 FLUSH MOUNT INSTALLATION:

- REQUIRED: Install a Gem Box electrical junction box at the desired location.

- See Figure - 01 for Steps 1 - 5.

FIGURE – 01

- Pull the wires from junction box

- Fasten the bare copper fixture wire to the green or bare copper wire in the box with a wire nut or fasten it to the MOUNTING PLATE with the ground screw provided.

- Fasten the white fixture wire to the white wire in the box with a wire nut. Tightly wrap the connection with electrical tape beginning 1” [25mm] below the wire nut to ensure the tape seals the end of the nut. Make sure that there are no exposed wires that could cause a short circuit.

- Repeat previous step with the black fixture wire and the black wire from the junction box. Make sure the wires are pushed into the junction box.

- Attach the MOUNTING PLATE to the FIXTURE BODY, and JUNCTION BOX using the MOUNTING SCREWS making sure to keep the proper alignment of the plate.

- See Figure - 02 for Step 6.

FIGURE – 02

- Use the METAL PLATE, and MOUNTING NUTS to secure the CANOPY, and GLASS SHADE to the ceiling.

- See Figure - 03 for Step 7.

- Assemble the GLASS DIFFUSER by moving the MOUNTING BRACKETS into the notches of the GLASS DIFFUSER, and secure MOUNTING BRACKETS with SET SCREWS

- See Figure - 04 for Step 8.

- Use the SET SCREWS to secure the GLASS DIFFUSER with the MOUNTING BRACKETS to the GLASS SHADE.

SERVICING THE LIGHT:

- Unscrew the SET SCREWS to gain access to the LIGHTBULB for cleaning and servicing.

- If further assistance is required, please contact Product Support at 1-800-927-2120 (8am-6pm EST).